Phase 3 Summary: Communication Module Implementation and Validation

- Objective

The objective of Phase 3 is to establish a stable, debuggable, and verifiable communication mechanism between the MCU and the PC, based on a custom frame-based protocol.

The system is required to support:

  - Custom frame structure design

  - Data integrity verification

  - Error frame detection and isolation

  - Communication recovery after session interruption

The primary goal of this phase is to build a reliable communication model, rather than chasing architectural complexity.

- TCP-Based Virtual Communication Validation

Before deploying the system on physical hardware, a TCP-based simulation was conducted to validate the communication protocol in a controlled environment.

Implementation

  - PC side: TCP Server

  - MCU behavior: simulated as TCP Client

  - Protocol reused from UART design

  - Frame structure:

    - SOF/LEN/TYPE/SEQ/CRC

  - Introduced periodic CRC error injection mechanism

Validation Results

  - Frame structure design was verified to ben correct

  - Error injection mechanism functioned as expected

  - Corrupted frames were successfully discarded

  - Communication continuity was preserved

  - Subsequent frames were processed correctly

This stage achieved a complete closed-loop validation in a software environment.

Significance

This step decouples:

Protocol correctness form hardware physical layer constraints

Allowing protocol logic to be fully verified before hardware integration.

- UART-Based Hardware Communication Implementation

After validation the protocol, it was migrated to the real hardware environment.

System Setup

  - MCU: STM32 NUCLEO-F401RE

  - PC: CH341 USB-TO-UART interface

  - Communication model: blocking, frame-by-frame reception

Functional Implementation

  - REQ-RESULT-LED control loop established

  - Successful end-to-end hardware communication

  - Stable system operation achieved

4.Error Injection and Robustness Testing

The CRC error injection mechanism was reintroduced in the UART environment.

Results

  - Corrupted frames were correctly detected and discarded

  - No deadlock occurred during communication

  - System continued running without interruption

  - LED behavior followed safe logic

Conclusion

The communication layer demonstrates strong error isolation capability.

Asynchronous Communication Attempt and Architectural Decision

Motivation

Although the blocking communication model proved to be stable, a critical limitation was observed:

After the session starts, re-establishing the connection (e.g., after PC terminal restart) may lead to communication blocking.

The MCU remains stuck in a blocking state

Manual intervention (pressing the RESET button) is required to recover the system

This behavior reduces system robustness and limits practical usability in real-world scenarios.

Attempt

To address this issue, an attempt was made to upgrade the system from:

Blocking communication-Asynchronous communication architecture

The goal was to:

  - Eliminate dependency on hand reset

  - Improve system responsiveness to connection interruptions

  - Enable more flexible communication handling

Observed Issues

However, during implementation, the following problems occurred:

  - LED did not respond

  - Conditional logic was not triggered

  - Frame parsing failed

  - Uncertainty in frame timing and buffer concatenation

  - Debugging complexity increased significantly

Evaluation

Based on the phase objectives:

  - The current priority is stability and verifiability, not architectural advancement

  - Asynchronous communication introduces higher system complexity

  - The root cause of the issue could not be quickly isolated

  - Performance improvement is not yet a system bottleneck

Decision

This system was rolled back to the blocking communication model

Additionally, instead of fully adopting an asynchronous architecture, a finite timeout mechanism (introduced in Section 6) was implemented as a low-complexity alternative solution.

This approach:

  - Resolves the infinite blocking issue

  - Enables automatic recovery after disconnection

  - Maintains system simplicity and debuggability

6.Finite Timeout Optimization

To address the limitation of blocking communication （potential infinite blocking after PC disconnecting）:

  - Improvement

    - Replaced HAL_MAX_DELAY with finite timeout

    - Upon timeout, control returns to main loop

  - Result

    - Automatic recovery after PC terminal restart

    - No manual MCU reset required

    - Improved robustness with minimal complexity increase

8.Achievements of Phase 3

  - TCP-based protocol validation

  - UART hardware migration

  - CRC error injection testing

  - Session recovery mechanism

  - Stable version archived

The system now satisfies

  - Stability

  - Debuggability

  - Error traceability

  - Portability

Conclusion

Phase 3 successfully achieves its objective:

A reliable and verifiable communication module has been established.

As the next step, the validated communication framework developed in Phase 3 will be migrated from the STM32F401 platform to the STM32H7 platform.

This migration is motivated by the increasing system requirements in the following stages, particularly for handling higher data throughput and integrating additional peripherals such as camera modules and display devices. While the F401 platform is sufficient for communication validation, it is not optimal for future expansion toward image data streaming and real-time processing.

Therefore, the H7 platform is selected as the target platform for subsequent development, while the current Phase 3 implementation is preserved as a stable and verified baseline.