Phase 2 Summary

The objective of Phase 2 was to introduce a real hardware communication link while preserving the stability of AI inference and decision logic, enabling a smooth transition form viryual validation to a real embedded system closed loop.

Phase Implementation Workflow

Closed-loop Validation in a Virtual Communication Environment

Before introducing real hardware, the AI inference closed loop was first validated in a PC-to-PC virtual communication environment. The main tasks included:

  - Constructing a request-response model based on TCP/virtual communication

  - Simulating the STM32 side to send REQ requests

  - On PC side, upon receiving a request:

    - Performing YOLO inference

    - Executing binary decision logic (YES/NO)

    - Returning a unified RESULT message format

The following aspects were verified:

  - Correctness of the inference pipeline

  - Stability of the decision logic

  - Feasibility of the communication format

The core of this stage was:

- To verify that the system logic itself is correct, without introducing unnecessary complexity

After completing the virtual stage, the related code was explicitly frozen and preserved as the logical baseline for subsequent hardware validation.

Closed-loop Validation in a Real Hardware Communication Environment (Hardware-in-the -loop)

Based on the successful virtual validation, the system was migrated to real hardware communication environment:

  - Establishing real serial communication using STM32+USB-USART (CH341)

  - The STM32 periodically sends REQ requests

  - On the PC side, the following frozen modules from the virtual validation stage were reused

    - AI inference module’

    - Binary decision module

  - The communication layer was switched from TCP/virtual communication to UART

Under real hardware-triggered conditions, the following were validated:

  - System stability

  - Response consistency

  - Hard behavior (LED response)

The focus of this sub-stage was to verify whether the system logic validated in the virtual environment can be seamlessly transferred to a real embedded system.

System Architecture and Resolution Strategies

The system architecture remains consistent between the virtual validation stage and the hardware validation stage.

The only difference lies in the communication layer implementation.

Issues Encountered and Resolution Strategies

During Phase 2, no issues were encountered in the virtual validation path. However, during hardware validation, the following categories of issues were identified:

- Configuration / project generation issues (IDE / peripheral mode settings)

- Resource conflicts (multiple components occupying the same serial port)

- Driver / OS-level issues (driver states, port number conflicts)

- Physical layer issues (cables, poor connections)

- Protocol / input detail issues (line endings, transmission methods)

- Python project structure / path issues (module import failures)

- Interface / naming consistency issues (incorrect API usage, mixed aliases)

- Refactoring leftovers / initialization placement issues (old and new logic coexisting)

- Debugging strategy issues (when system behavior becomes unpredictable, reverting to a known stable version and reintroducing variables incrementally)

The solutions to these issues have been documented in the corresponding markdown files.

Phase Outcomes

By the end of Phase 2, the system achieved:

  - A real hardware closed loop:STM32-PC-AI-STM32

  - Stable influence of AI inference results on hardware behavior

  - A clear system structure with frozen interfaces and predictable states

  - Core code and execution results fully archived

Conclusion

Phase 2 successfully completed the transition from a virtual validation environment to a real hardware communication environment, achieving the first real closed-loop integration of AI inference withi an embedded system.

The current system now provides

  - Basic semantics suitable for long-running service operation

  - Clear module boundaries

  - A solid structural foundation for advancing to Phase 3

Phase 2 is hereby formally concluded.