Phase 4 Summary

Image Transmission + AI Closed Loop + Robustness Validation

Phase Objective

In Phase 3, although a stable communication closed loop between PC and MCU over UART was successfully established, the loop remained essentially a simplified result-loop:

  - The transmitted data was limited to AI inference result (YES/NO);

  - The actual business data (image data) was not included in the communication pipeline.

Therefore, Phase 3 validated:

  - The frame protocol can reliably transmit control results;

  - The MCU can respond based on received results.

However, it did not validate:

  - Whether the frame structure can support image data;

  - Whether image data can be correctly transmitted, received, parsed, and reconstructed;

  - Whether the system truly forms a complete data loop:

Image → Communication → AI → Result → MCU

In other words:

The Phase 3 system was functionally valid, but still a simplified abstraction of the real application scenario.

So, the core of this Phase is: upgrade the system form “Result-only closed loop” to “Full data closed loop with real image transmission”.

Specifically:

  - Frame-based transmission of image data;

  - Multi-channel communication architecture (TCP + UART);

  - Integration of image data into AI inference pipeline;

  - Robustness validation under abnormal communication conditions.

Key principle:

Focus on achieving a realistic and complete data loop, not on introducing unnecessary architectural complexity.

Dual-Channel Communication Architecture

Target Architecture (Final Form)

MCU Side:

  - Capture image via camera (JPEG);

  - Send image to PC.

PC Side:

  - Receive image;

  - Performs AI inference;

  - Send result back to MCU.

Data flow:

JPEG → Communication → AI → RESULT → MCU

This represents the true engineering form of the system.

Limitation of Phase 3

Phase 3 only transmitted:

AI inference result (YES/NO).

Instead of:

Raw data (image).

Thus, it was essentially:

A result-driven simplified loop.

It could not verify:

  - Large data transmission capability;

  - Image integrity;

  - Frame protocol scalability.

Current alternative architecture

Since camera hardware was not yet integrated, a simulation approach was introduced:

  - A sender module reads local image files;

  - Sends image data using frame structure.

System flow:

Sender → Communication → AI → RESULT → MCU

However, during implementation, a communication interface conflict issue was exposed.

During the implementation process, I tried to use the same serial port for communication between the MCU and the communication module, and for communication between the sender and the communication module.

Problem encountered:

  - PC communication service occupied UART;

  - Sender attempted to use the same UART;

  - Resource conflict occurred.

Architecture Adjustment

Minimal modification:

  - Image transmission: TCP;

  - Control/result transmission: UART.

New data flow:

Image Sender → TCP → Communication Model → UART → MCU

This architecture serves as a temporary substitute to simulate the data flow of the final system.

Image Transmission Mechanism

Initial Approach: Single-Frame JPEG

In the early stages of this phase, image data is transmitted in the most direct way:

Entire JPEG sent as one frame (TYPE_IMAGE).

Advantage:

  - Simple;

  - Easy validation.

Result:

  - Works correctly;

  - AI inference successfully triggered.

Limitations:

  - Frame size too large;

  - Not suitable for embedded constraints;

  - No fine-grained error control.

Chunked Transmission Design

So, Image chunking mechanism was introduced:

  - Fixed-size chunks (e.g., 256 bytes);

  - Multi-frame transmission (TYPE_IMAGE_CHUNK);

  - Implemented at application layer (no change to base protocol).

Sender validation:

  - JPEG → chunks → multi-frame send;

  - Example: 5073 bytes → 20 chunks;

  - Stable TCP transmission.

Receiver Reassembly Validation

In the early stages, in order to validate the correctness of the chunking mechanism, a standalone validation module was created:

  - Receives TCP stream;

  - Parses frames;

  - Reassembles images;

  - Saves locally.

Excludes:

  - AI inference;

  - UART;

  - MCU.

Results:

  - 8 images successfully reconstructed;

  - Images are valid and viewable.

Conclusion:

Chunk transmission → reception → reassembly is fully validated

AI Closed Loop Integration

Updated data flow:

Chunk frames → Reassembly → JPEG → YOLO → YES/NO → UART → MCU

Results:

  - AI inference triggered correctly;

  - Correct YES/NO classification;

  - MCU successfully controls LED.

Conclusion:

Fully system closed loop successfully established.

Environment & Engineering Stability

Python Environment

Problem:

Multiple interpreters → inconsistent execution

Solution:

  - Unified environment;

  - Fixed VS Code interpreter.

Traceability Fix

Problem:

Phase 1 test scripts (AI binary test) not runnable.

Root cause:

Import path conflicts

Result:

  - Phase 1 validation chain restored;

  - AI link successfully repaired;

  - The system has restored traceability and stable operation capabilities.

Error Injection & Robustness Testing

After completing the image chunk transmission and reassembly mechanism, this phase did not directly equate “functional correctness” with “system reliability” Instead, in order to simulate the operation under real-world conditions, further validation was carried out focusing on system behavior abnormal communication conditions.

The purpose of this stage is not merely to demonstrate that the system works under ideal conditions, but to verify the following:

  - Whether the system can correctly detect corrupted data during transmission;

  - Whether invalid data would be incorrectly forwarded into the AI inference pipeline;

  - Whether the system can recover and continue processing subsequent images after a failure;

  - Whether the chunk reassembly mechanism maintains correct internal state under incomplete Input conditions;

In essence, the core concern of this section is:

Whether the system remains controllable under the realistic assumption of unreliable input.

Initial Approach: CRC Injection (Communication Side)

In earlier stages, error injection was implemented within the PC-side communication module. This approach was initially retained in this phase.

Specifically:

  - Image data had already entered the communication module;

  - CRC values were intentionally modified during processing;

  - The receiver would detect the mismatch and discard the corrupted frame;

The purpose of this test was to validate:

  - Whether the frame protocol supports CRC-based error detection;

  - Whether corrupted frames can be correctly identified and discarded;

  - Whether such frames would continue processing subsequent valid frames after encountering errors.

The result showed that:

  - Corrupted frames were correctly detected;

  - Corrupted frames were not forwarded to AI inference;

  - Subsequent frames could still be processed normally;

  - The MCU-side closed loop remained functional.

This indicates that the communication protocol possesses basic error isolation capability when error occur within the communication module.

However, further analysis revealed a critical limitation of this approach:

Mismatch between injection location and real-world errors.

In real system, errors rarely originate from within the communication module itself. Instead, they are more likely to occur before data enters the communication layer.

Typical sources include:

  - Camera acquisition or encoding errors;

  - Corrupted data generated at the sender side;

  - Transmission instability leading to partial data loss or corruption;

  - Interface instability causing multiple consecutive frames to become invalid.

So, in real scenarios, the communication module typically receives already corrupted input, rather than generating corrupted frames internally.

Therefore, while CRC injection within the communication module is useful for validating protocol-level error detection, it does not accurately represent real-world failure conditions.

Adjustment: Moving Error injection to Sender Side

Based on this observation, the error injection strategy was revised:

Error injection was moved from the communication module to the sender side.

This adjustment ensures that:

The communication module processes realistically corrupted input data, rather than artificially generated internal errors.

As a result, the testing scenario becomes significantly closer to actual deployment conditions.

Error Model Abstraction: Consecutive Frames

After relocating the error injection point, another key question arises:

How should real-world communication failures be abstracted in to a practical and testable model?

Instead of enumerating all possible error types, this phase adopts a generalized abstraction:

Consecutive invalid frames

This abstraction is chosen because:

- It captures the observable effects of various sources

  - Regardless of whether errors originate from camera, encoding, or transmission, they often manifest as:

    - Multiple corrupted frames;

    - Missing frames;

    - Continuous segments of unstable data;

- It better reflects real communication degradation;

  - In practice, errors are rarely isolated events; they often in bursts over a period of instability.

- It directly targets the core robustness of the chunk-based system

  - The critical challenge is not whether a single corrupted frame can be detected, but:

    - What happens when multiple chunks of an image are missing or corrupted;

    - Whether incomplete images are mistakenly reconstructed;

    - Whether reconstruction state affects subsequent images.

Based on this reasoning, this phase defines:

Consecutive invalid frames as the primary error model for robustness validation.

Test Scenario Design

4 representative scenarios were constructed:

- Corruption at the beginning of an image:

  - Verifies whether the system incorrectly initializes reconstruction state;

  - Ensures no false image completion occurs.

- Corruption in the middle of an image:

  - Verifies whether missing critical chunks lead to incorrect “image complete” signals;

  - Ensures incomplete images are not passed to AI.

- Corruption at the end of an image:

  - Verifies whether near-complete images trigger premature completion;

  - Particularly sensitive to boundary condition errors.

- Corruption across image boundaries:

  - Verifies whether error states propagate to subsequent images;

  - Ensures proper state reset and isolation.

Excluded Case: Frame Boundary Corruption

Another class of potential failures was identified:

Frame boundary corruption

This includes:

  - Frames header corruption;

  - Length field misalignment;

  - Stream desynchronization during frame parsing.

Unlike chunk-level corruption, these issues affect the fundamental correctness of the protocol layer.

The objective of this phase are:

  - Validate chunk-based image transmission;

  - Verify reconstruction and recovery logic;

  - Establish baseline system robustness.

It was decided that: Frame boundary corruption is excluded from this phase and reserved for future protocol-level validation.

Validation Result and Engineering Conclusions

Experimental results demonstrate that under all tested scenarios:

  - Corrupted chunks are correctly detected and discarded;

  - Incomplete images do not trigger reconstruction completion;

  - Invalid data is never forwarded to the AI inference pipeline;

  - Internal state remains consistent and properly managed;

  - The system successfully recovers and processes subsequent images.

Therefore, it can be concluded that:

  - The system supports not only single frame error detection;

  - But also maintains correctness under consecutive communication failures;

  - And demonstrate reliable recovery behavior after image-level failure.

PC-side Lightweight Optimization

Communication-AI Decoupling and Buffer Mechanism

During system operation, a class of instability issues was observed, characterized by occasional abnormal interruption of the program.

Further analysis revealed that:

  - Both the MCU side and the PC side communication module adopted blocking communication mechanisms;

  - Under certain conditions, blocking behavior on both sides could interact and amplify each other;

  - This occasionally led to system interruption, with symptoms similar to unexpected termination.

The root cause could not be immediately identified, which made debugging difficult.

In practice, the recovery behavior showed clear uncertainty:

  - In some cases, the issue originated from the MCU side, and the system could recover after pressing the RESET button.

  - In other cases, the issue was related to the PC communication module, requiring the terminal to be restarted and the program relaunched.

This ambiguity made it difficult to determine whether failures were caused by:

  - MCU blocking;

  - PC communication issues;

  - Or interference from the AI inference process.

Subsequent investigation confirmed that:

The primary cause of these abnormal interruptions was that the error injection mechanism on the communication side had not been disabled, resulting in continuous disturbance to the communication process.

However, despite being caused by a testing configuration issue, this behavior exposed an important structural limitation:

In a fully blocking and tightly coupled pipeline, disturbances are easily amplified, and failure sources become difficult to isolate.

Original Processing Flow

Before optimization, the PC-side processing followed a strictly linear pipeline:

Receive → Reassemble → AI Inference → Generate RESULT → Send to MCU

In this structure：

  - Communication and AI inference were executed in the same path;

  - Any delay or blocking in AI inference would directly affect communication;

  - Under unstable conditions, the entire pipeline could stall or behave unpredictably.

To solve this issue, a structural adjustment was made:

Introduce a buffer queue to decouple communication and AI inference.

The updated workflow becomes:

  - Communication module:

    - Receive data from sender;

    - Perform chunk reassembly;

    - Push reconstructed JPEG into a buffer queue

  - AI module:

    - Independently fetch images from the queue;

    - Perform inference;

    - Output results.

Effect of Optimization

After introducing the buffer mechanism:

  - Communication is no longer blocked by AI inference latency;

  - The system becomes more stable under continuous data input;

  - Failure sources become easier to isolate;

  - Overall behavior is more predictable compared to the original linear pipeline.

Although the improvement is moderate rather than drastic, it provides a clear structural advantage over the previous design.

Auto Serial Detection

This mechanism was introduced following an issue observed during a previous project demonstration, where a driver update caused the serial port number to change, leading to communication failure. To avoid such dependency on manual configuration, automatic serial port detection was implemented

Phase Conclusion

This phase does not aim to introduce new system functionalities, but rather to resolve key uncertainties required for subsequent development.

Through this phase:

  - The feasibility of transmitting image data over the existing frame-based communication structure has been validated;

  - The chunk-based transmission and reassembly mechanism has been verified as correct and reliable;

  - The system has successfully transitioned from a result-driven loop to a full data loop with image input;

  - The AI inference pipeline has been integrated into the communication flow and validated in practice;

  - The system behavior under abnormal communication conditions has been systematically tested and confirmed to be controllable and recoverable；

  - The PC-side architecture has been optimized, improving overall system robustness.

This baseline ensures that:

  - The current architecture is capable of supporting real image data;

  - The communication protocol is robust under realistic failure conditions;

  - The system can safely proceed to hardware integration without introducing unknown risks.

The system architecture has been fully validated using a simulated data source, and subsequent work mainly involves replacing the data source with real hardware under practical constraints.

Based on these results, the system is now ready for the next stage:

  - Migration to the STM32H7 platform；

  - Integration of real camera hardware；

  - Full system validation under physical conditions.