Phase 5 Summary

- Overview

The main objective of this phase was to complete the migration and validation of the existing system from the STM32F401 platform to the STM32H743 platform. In parallel, a structural refactoring of the software architecture was carried out during the migration process.

Unlike previous phases, this phase does not focus on adding new functionality. Instead, its primary goal is:

To reorganize the software structure through modularization while preserving system behavior, thereby improving maintainability and scalability.

- Migration and Validation Process

The migration was performed in a step-by-step manner following a progressive validation strategy:

  - Verification of the minimal system on the H7 platform (GPIO/LED/KEY);

  - Establishment and validation of the UART communication link;

  - Reconstruction of the Phase 2 PC-MCU closed loop (REQ/RESULT);

  - Migration of the frame-based communication and dual-channel architecture (TCP+UART);

  - Lightweight error injection testing to evaluate system robustness.

The results indicate that:

  - The PC-side required no modification and remained fully compatible with the H7 platform;

  - Communication, AI inference, and MCU response all function correctly;

  - The system is able to recover from abnormal conditions.

These results demonstrate that the migration introduces no new system-level issues and that the existing architecture exhibits good platform adaptability.

Observation During Validation

During system validation, a slight LED flickering phenomenon was observed under continuous polling mode.

Analysis shows that this behavior is caused by a timing mismatch between the AI inference result update on the PC side and REQ polling cycle on the MCU side. Around the update boundary, the MCU may receive different results (old vs new), leading to temporary LED state switching.

This phenomenon is not a communication or logical error and does not affect system functionality or result interpretation. With the current dual-LED semantics (red/green), the system remains sufficiently intuitive.

From an engineering perspective, this behavior is a normal side effect of a polling-based architecture. It is expected to be naturally eliminated in the future “button-triggered acquisition system” stage, and there no additional optimization is performed at this stage.

- Core Work: Software Architecture Refactoring

  - Motivation

In the F401 stage, although the system functionality was fully validated, a significant structural issue was identified:

All logic was concentrated in main.c, resulting in excessive code complexity and poor maintainability.

Specifically:

  - Communication logic, protocol handling, and hardware control were tightly coupled;

  - No clear separation between logic and hardware layers;

  - Local modifications could affect the entire system;

  - Debugging and readability costs increased significantly.

As system complexity grew (frame protocol, image transmission, AI pipeline), this issue became a major bottleneck.

Therefore, during the migration, the following decision was made:

Use platform migration as an opportunity to perform a systematic refactoring and introduce a modular architecture.

  - Layered Architecture Design

The refactored system is organized as follows:

main (control layer)

↓

uart_service (communication logic)

↓

uart_frame (protocol layer)

↓

HAL (hardware abstraction layer)

bsp_led (hardware abstraction module)

Module responsibilities:

  - main.c:

    - Handle system initialization and scheduling only.

  - uart_service:

    - Manages communication flow (REQ sending/RESULT receiving);

    - Implements application-level logic (YES/NO parsing).

  - uart_frame:

    - Handles frame encapsulation and parsing (SOF/LEN/CRC/EOF);

    - Abstracts low-level communication details.

  - bsp_led:

    - Provides hardware abstraction for LED control;

    - Fully decoupled from communication logic.

  - Refactoring Results

Decoupling of System Components

From:

main.c (fully coupled)

To:

control + communication + protocol + hardware abstraction

Each module now has clearly defined responsibilities.

Improved Maintainability

  - Communication logic can be modified without affecting hardware code;

  - Hardware components can be replaced independently (e.g., LED → buzzer);

  - The main function is simplified and easier to understand.

Enhanced Scalability

The new structure directly supports future extensions:

  - Camera module integration (replacing current data source);

  - Button triggered acquisition;

  - Additional peripherals (OLED display, buzzer etc.).

Engineering Maturity

This refactoring marks the transition from:

“Function-oriented prototype” → “Structured engineering system”

- Current System Status

After migration and refactoring, the system has reached the following state:

  - Stable operation on STM32H743;

  - Reliable PC-MCU communication loop;

  - Functional frame-based communication;

  - Stable image transmission and AI inference;

  - Correct LED feedback behavior;

  - Basic system robustness verified.

- Conclusion

In this phase, the platform migration was successfully completed, and a modular software architecture was established, resolving the excessive coupling in the original system.

As a result:

  - The system structure is significantly improved;

  - Module responsibilities are clearly defined;

  - Future development complexity is reduced;

Furthermore, the migration results confirm that:

The current system architecture is stable, portable, and suitable for further functional expansion.

- Next Steps

The next phase will focus on transitioning from a simulated data source to a real hardware input:

Button trigger → Camera capture → Image transmission → AI inference → MCU feedback

The goal is to validate the system under real-world conditions and complete the final system integration.